# Switch Abstraction Interface

# Change Proposal

| Title | Subport interface , .1D Bridges , bridge port interface |
| --- | --- |
| Authors | Mellanox |
| Status | In Review |
| Type | Standards Track |
| Created | 07/18/2016 |
| SAI-Version | 1.0 |

1	Overview	3

2	Sub port interface	3

2.1	New port type attribute	3

2.2	Exmple - Router sub port flow	4

3	Non Vlan aware bridge domains (.1D bridges)	4

3.1	Non Vlan aware bridge flows	5

4	Bridge ports	5

4.1	SAI bridge/router object current state	5

4.2	SAI bridge/router object proposed state	6

4.3	Bridge port type types	6

4.3.1	Bridge port type - Port	7

4.3.2	Bridge vport type – { Port , vlan}	7

4.3.3	Bridge port type- router	7

4.3.4	Bridge port type – tunnel	8

4.4	Bridge attributes	9

5	Specification	9

5.1	SAI port object	9

5.2	SAI FDB object	9

5.3	SAI STP object	9

5.4	SAI router interface object	9

5.5	SAI bridge port object	10

5.5.1	Bridge port  API summary	11

5.6	SAI bridge object	11

5.6.1	Bridge API summary	12

6	Examples	12

7	References	12

# Overview

The proposal contain three prats

- In order build a discrete pipeline and well define layers additional set of object are needed Bridge ports, Currently when entering the switch we have -> ingress port /LAG When entering a router we have SAI RIF A bridge port object is missing

- increase SAI 4k vlan broadcast domain  by adding multiple vlan unaware bridges (.1D)  to te existing Vlan aware bridge (1.Q)

- add an ability to define an interface base on {port,vlan} were interface can be router interface or bridge interface

# Sub port interface

Ability to define an interface base on {port,vlan} were interface can be router interface or bridge interface

## New port type attribute

Control on the port mode optional modes

- Port – legacy

- Sub,port- port is splitter into logical interfaces each interface is define by {port,vlan}

## Exmple - Router sub port flow

# Non Vlan aware bridge domains (.1D bridges)

Increase SAI 4k vlan broadcast domain by adding multiple vlan unaware bridges (.1D)  to the existing Vlan aware bridge (1.Q)

Add ability to create .1D bride object

Add ability to bind  an "interface" to the a .1D bridge

- only interface type {port,vlan}

## Non Vlan aware bridge flows

# Bridge ports

In order build a discrete pipeline and well define layers additional set of object are needed Bridge ports, Currently when entering the switch we have -> ingress port /LAG When entering a router we have SAI RIF A bridge port object is missing

## SAI bridge/router object current state

## SAI bridge/router object proposed state

## Bridge port type types

Port – represent phy port or LAG traditional .1Q bridge port

Vport - represent phy port or LAG. vlan   .1Q bridge port  interface

Router - represent the port that connect the bridge to the router

Tunnel - represent the port that connect the bridge to tunnel

.

### Bridge port type - Port

Represent phy port or LAG traditional .1Q bridge port

Can be added into .1Q Bridge only

Can be binded only form a port mode {port}

By default all port will have a corresponding bride port add to the default 1.q bridge

Valid attribute:

### Bridge vport type – { Port , vlan}

Represent phy port or LAG .vlan

Can be added into .1D Bridge only

Can be binded only form a port mode {vport}

### Bridge port type- router

Represent to bridge port connect to the router

Two types of router bridge port

1.Q router bridge port- a single port representing all router Vlan interfaces

Per bridge 1.D router bridge port   per 1.D bridge port

### Bridge port type – tunnel

Represent to bridge port connect to a tunnel

There is one to one mapping between the bride port and the tunnel

## Bridge attributes

All bridge related attribute will move from port/LAG to bridge port object

Bridge attribute

STP

Vlan objects

…

# Specification

## SAI port object

typedef enum _sai_port_bind_attr_t

{

SAI_PORT_MODE,

SAI_SUB_PORT_MODE,

} sai_port__bind_attr_t ;

typedef enum _sai_port_attr_t

{

//new attribute

/** port type[sai_port_bind_attr_t] */

SAI_PORT_BIND_MODE,

//removed atrtributes

SAI_PORT_ATTR_FDB_LEARNING,

SAI_PORT_ATTR_MAX_LEARNED_ADDRESSES,

SAI_PORT_ATTR_FDB_LEARNING_LIMIT_VIOLATION ,

} sai_port_attr_t ;

## SAI FDB object

typedef enum _sai_fdb_entry_attr_t

{

//new attribute

SAI_FDB_ENTRY_ATTR_BRIDGE_PORT_ID

//removed attributes

SAI_FDB_ENTRY_ATTR_PORT_ID

}

## SAI STP object

TBD

## SAI router interface object

TBD

typedef enum _sai_vis_collector_type_attr_t

{

SAI_VIS_COLLECTOR_TYPE_PERIODIC,

SAI_VIS_COLLECTOR_TYPE_ON_DEMAND

## SAI bridge port object

typedef enum _sai_bridge_port_attr_t

{

TBD

} sai_bridge_port_attr_t ;

/**

* @brief Create bridge port

*

* @param[out]  bridge_port_id

* @param[in] 	 attr_count number of attributes

* @param[in] 	 attr_list array of attributes

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_create_bridge_port_fn)(

_Out_ sai_object_id_t* bridge_port_id,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

);

/**

* @brief Remove bridge port

*

*  @param[in] bridge_port_id.

*

*  @return  SAI_STATUS_SUCCESS on success

*           Failure status code on error

*/

typedef sai_status_t (*sai_remove_brideg_port_fn) (

_In_  sai_object_id_t   brdge_port_id

);

/**

* @brief Set attributes for bridge port

*

* @param[in] bridge port id

* @param[in] attr attribute to set

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_set_bridge_port_attribute_fn)(

_In_ sai_object_id_t  bridge_port_id,

_In_ const sai_attribute_t *attr

);

/**

* @brief  Get attrbutes of bridge port

*

* @param[in] bridge_port_id

* @param[in] attr_count  number of attributes

* @param[inout] attr_list  array of attributes

*

* @return SAI_STATUS_SUCCESS on success

*        Failure status code on error

*/

typedef sai_status_t (*sai_get_bridge_port_attribute_fn)(

_In_ sai_object_id_t   bridge_port_id ,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

### Bridge port  API summary

typedef struct  bridge_port_t

{

sai_create_bridge_port_fn         			create_bridge_port;

sai_remove_bridge_port_fn 			remove_bridge_port;

sai_set_bridge_port_attribute_fn  			set_bridge_port_attribute;

sai_get_bridge_port_attribute_fn  			get_bridge_port_attribute;

} sai_bridge_port_api_t;

## SAI bridge object

typedef enum _sai_bridge_attr_t

{

TBD

} sai_bridge_attr_t ;

/**

* @brief Create bridge

*

* @param[out]  bridge_port_id

* @param[in] 	 attr_count number of attributes

* @param[in] 	 attr_list array of attributes

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_create_bridge_fn)(

_Out_ sai_object_id_t* bridge_id,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

);

/**

* @brief Remove bridge

*

*  @param[in] bridge_id.

*

*  @return  SAI_STATUS_SUCCESS on success

*           Failure status code on error

*/

typedef sai_status_t (*sai_remove_brideg_fn) (

_In_  sai_object_id_t   brdge_id

);

/**

* @brief Set attributes for bridge

*

* @param[in] bridge id

* @param[in] attr attribute to set

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_set_bridge_attribute_fn)(

_In_ sai_object_id_t  bridge_id,

_In_ const sai_attribute_t *attr

);

/**

* @brief  Get attrbutes of bridge

*

* @param[in] bridge_id  bridge_id

* @param[in] attr_count  number of attributes

* @param[inout] attr_list  array of attributes

*

* @return SAI_STATUS_SUCCESS on success

*        Failure status code on error

*/

typedef sai_status_t (*sai_get_bridge_attribute_fn)(

_In_ sai_object_id_t   bridge_id ,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

### Bridge API summary

typedef struct  bridge_t

{

sai_create_bridge_fn         				create_bridge;

sai_remove_bridge_fn 				remove_bridge;

sai_set_bridge_attribute_fn  			set_bridge_attribute;

sai_get_bridge_attribute_fn  			get_bridge_attribute;

} sai_bridge_api_t;

# Examples

# References